WECC Historical Hydropower Generation Data, 2018-2022

Data overview  |  Prepared by Yi Lu  |  2026-10-03

# 1. Purpose and scope

This folder contains historical hydropower generation data for the dams marketed by the Bonneville Power Administration (BPA) and the Western Area Power Administration (WAPA), collected to support reconstruction of node-level hydro output in the WECC 240-bus model. The dam and project list was provided by David Chassin.

BPA dams: 14 Federal Columbia River Power System dams, including the ten "Big 10" run-of-river dams; hourly, dam-level, 2018-2022.

WAPA dams: plant generation for the five WAPA projects (Boulder Canyon, Parker-Davis, Colorado River Storage Project, Pick-Sloan Missouri Basin Program, Central Valley Project), 2018-2022; hourly where publicly available, otherwise monthly.

Only public data is used. Values are stored as published by the source agency (no unit conversion, gap filling or outlier removal). For the BPA dams an added column, power_MW_adjust, corrects 7 obvious recording errors (an extra digit or a misplaced decimal point); the published power_MW is kept next to it. All hourly timestamps are UTC, hour-ending.

# 2. Folder contents

| Path | Content |
| --- | --- |
| WECC_Hydro_Data_Overview.docx | This overview |
| VERIFICATION_REPORT.docx | Accuracy checks of all data files against the source agencies |
| BPA_dams/BPA_dams_README.docx | Documentation of the BPA data: dams, columns, sources, checks, known issues |
| BPA_dams/dam_list.csv | The 14 dams: owner, river, state, Big 10 flag, EIA plant id, nameplate capacity, data file |
| BPA_dams/manual_download_links.txt | Direct USACE download link for every dam |
| BPA_dams/power_MW_adjustments.csv | Change log: the 7 hours where power_MW_adjust differs from power_MW, with original and adjusted values |
| BPA_dams/USACE_CWMS_hourly/ | One hourly CSV per dam, plus data_dictionary.csv and series_used.csv (exact source series) |
| WAPA_dams/WAPA_dams_README.docx | Documentation of the WAPA data: projects, plants, sources, checks, known issues |
| WAPA_dams/plant_list.csv | All 45 plants with project, owner, state, division and data file |
| WAPA_dams/sources.csv | Web page and direct download link of every source series or file |
| WAPA_dams/data_dictionary.csv | Column definitions and units of every WAPA file type |
| WAPA_dams/1_Boulder_Canyon/ ... 5_Central_Valley/ | Data files, one folder per WAPA project |

# 3. BPA dams (folder BPA_dams/)

Data from the USACE Northwestern Division CWMS database, including the USBR dams Grand Coulee and Hungry Horse: 14 dams, hourly, 2018-2022, UTC, hour-ending, one CSV per dam (43,824 rows).

| Source | Link / note |
| --- | --- |
| USACE Dataquery (web interface) | https://www.nwd-wc.usace.army.mil/dd/common/dataquery/www/ |
| Download service | .../web_service/webexec/getjson Revised (REV) and raw (RAW) series were both retrieved, REV preferred. Plant power REV is empty for 13 dams in 2018-2022, so power uses RAW (Hungry Horse: REV). |
| Direct link per dam | BPA_dams/manual_download_links.txt |

| Code | Dam | Owner | River | Big 10 | File |
| --- | --- | --- | --- | --- | --- |
| ALF | Albeni Falls | USACE | Pend Oreille River, ID |  | ALF_AlbeniFalls_hourly_2018_2022_UTC.csv |
| BON | Bonneville | USACE | Columbia River, WA/OR | yes | BON_Bonneville_hourly_2018_2022_UTC.csv |
| CHJ | Chief Joseph | USACE | Columbia River, WA | yes | CHJ_ChiefJoseph_hourly_2018_2022_UTC.csv |
| DWR | Dworshak | USACE | North Fork Clearwater River, ID |  | DWR_Dworshak_hourly_2018_2022_UTC.csv |
| GCL | Grand Coulee | USBR | Columbia River, WA | yes | GCL_GrandCoulee_hourly_2018_2022_UTC.csv |
| HGH | Hungry Horse | USBR | South Fork Flathead River, MT |  | HGH_HungryHorse_hourly_2018_2022_UTC.csv |
| IHR | Ice Harbor | USACE | Snake River, WA | yes | IHR_IceHarbor_hourly_2018_2022_UTC.csv |
| JDA | John Day | USACE | Columbia River, WA/OR | yes | JDA_JohnDay_hourly_2018_2022_UTC.csv |
| LIB | Libby | USACE | Kootenai River, MT |  | LIB_Libby_hourly_2018_2022_UTC.csv |
| LGS | Little Goose | USACE | Snake River, WA | yes | LGS_LittleGoose_hourly_2018_2022_UTC.csv |
| LWG | Lower Granite | USACE | Snake River, WA | yes | LWG_LowerGranite_hourly_2018_2022_UTC.csv |
| LMN | Lower Monumental | USACE | Snake River, WA | yes | LMN_LowerMonumental_hourly_2018_2022_UTC.csv |
| MCN | McNary | USACE | Columbia River, WA/OR | yes | MCN_McNary_hourly_2018_2022_UTC.csv |
| TDA | The Dalles | USACE | Columbia River, WA/OR | yes | TDA_TheDalles_hourly_2018_2022_UTC.csv |

## 3.1 Columns

| Column | Description | Unit | Dams with data |
| --- | --- | --- | --- |
| power_MW | Plant total generation, hourly | MW | all 14 |
| power_MW_adjust | Plant total generation after correcting obvious recording errors: identical to power_MW except 7 hours, listed in BPA_dams/power_MW_adjustments.csv. Added by the data compiler. | MW | all 14 |
| station_service_MW | Station service (plant auxiliary) power, hourly | MW | ALF BON CHJ DWR GCL HGH IHR JDA LGS LWG LMN MCN TDA |
| spinning_reserve_MW | Spinning reserve carried by the plant, instantaneous at the hour | MW | ALF BON CHJ GCL HGH JDA LIB LWG TDA |
| capacity_available_MW | Available generating capacity, hourly | MW | GCL |
| outflow_kcfs | Total outflow (generation + spill + other), hourly average | kcfs (1,000 cubic feet per second) | all 14 |
| genflow_kcfs | Flow through turbines (powerhouse flow), hourly average | kcfs (1,000 cubic feet per second) | all 14 |
| spill_kcfs | Spillway flow, hourly average | kcfs (1,000 cubic feet per second) | all 14 |
| inflow_kcfs | Reservoir inflow, hourly average | kcfs (1,000 cubic feet per second) | DWR |
| forebay_elev_ft | Forebay (reservoir) water surface elevation, instantaneous at the hour | ft | all 14 |
| tailwater_elev_ft | Tailwater water surface elevation, instantaneous at the hour | ft | all 14 |
| storage_kaf | Reservoir storage, instantaneous at the hour | kaf (1,000 acre-feet) | BON CHJ DWR GCL HGH IHR LIB LGS LWG LMN |

# 4. WAPA dams (folder WAPA_dams/)

Generation data only, one folder per WAPA project; the project folders contain only data files.

| Folder | Plants | Best resolution | Source |
| --- | --- | --- | --- |
| 1_Boulder_Canyon | Hoover | Monthly | USBR RISE |
| 2_Parker_Davis | Davis, Parker | Monthly | USBR RISE |
| 3_CRSP | Blue Mesa, Crystal, Flaming Gorge, Fontenelle, Glen Canyon, Morrow Point | Hourly (2018-01 to 2022-04) + monthly (2018-2022) | WAPA SCADA + USBR RISE |
| 4_Pick_Sloan | USBR: Alcova, Big Thompson, Boysen, Buffalo Bill, Canyon Ferry, Estes, Flatiron Unit 3 (pump-generating), Flatiron Units 1-2, Fremont Canyon, Glendo, Green Mountain, Guernsey, Heart Mountain, Kortes, Marys Lake, Pole Hill, Seminoe, Shoshone, Spirit Mountain, Yellowtail. USACE mainstem: Big Bend, Fort Peck, Fort Randall, Garrison, Gavins Point, Oahe | Monthly | USBR RISE + EIA-923 (USACE mainstem) |
| 5_Central_Valley | Folsom, Judge Francis Carr, Keswick, Lewiston, New Melones, Nimbus, Shasta, Spring Creek, Stampede, Trinity | Monthly | USBR RISE |

| Source | Link / note |
| --- | --- |
| USBR RISE (monthly generation) | https://data.usbr.gov/ All 39 USBR plants: net and gross generation, station service, installed capacity, 2018-01 to 2022-12 |
| WAPA CRSP SCADA archive (hourly generation) | WAPA historical power plant data Hourly MW of the six CRSP plants (published as PDF, converted to CSV). WAPA has posted data up to April 2022. |
| EIA-923 (monthly net generation) | https://www.eia.gov/electricity/data/eia923/ Only for the six USACE Pick-Sloan mainstem dams: USACE does not publish their generation and they are not in RISE |

| CRSP hourly file | Hours in period | Hours with data | Coverage |
| --- | --- | --- | --- |
| Blue Mesa | 37,944 | 33,790 | 89.1% |
| Crystal | 37,944 | 33,953 | 89.5% |
| Flaming Gorge | 37,944 | 33,593 | 88.5% |
| Fontenelle | 37,944 | 33,305 | 87.8% |
| Glen Canyon | 37,944 | 33,206 | 87.5% |
| Morrow Point | 37,944 | 33,939 | 89.4% |

Time: hourly files are UTC, hour-ending (the WAPA SCADA clock is Mountain Standard Time all year; +7 h applied). Monthly values belong to the plant's local calendar month and cannot be re-cut to UTC months, so the month is kept and its UTC start is given (period_start_utc). Hours that are blank in the WAPA PDFs are left empty.

# 5. Accuracy checks (details in VERIFICATION_REPORT.docx)

Every check downloads the data again from the publishing agency; no file is compared with itself.

| Check | Result |
| --- | --- |
| BPA: all 14 direct download links re-downloaded (USACE CSV service), value by value | 5,029,861 values, 0 differences |
| BPA: CWMS The Dalles outflow vs USGS gauge 14105700 below the dam | Best lag 0 h (r = 0.87): UTC and hour-ending labels confirmed |
| BPA: annual energy of 14 dams vs EIA-923 net generation | Most dam-years within +/-1.5 %; exceptions in section 7 |
| WAPA: every RISE series re-downloaded through its CSV link, value by value | 157 series, 9,420 values, 0 differences (all links work) |
| WAPA: CRSP hourly generation summed by month vs USBR monthly gross generation (complete months) | Median ratio 1.000 for all six plants |
| WAPA: Flaming Gorge hourly generation vs USGS gauge below the dam, winter and summer | Best lag 0 h in both seasons: MST to UTC conversion confirmed |
| All hourly data: screening for recording errors (power vs measured flow / release) | 7 BPA values corrected in power_MW_adjust (section 6); no CRSP value meets the correction rule |
| WAPA: RISE annual net generation vs EIA-923 | Ratio 1.000 for almost all plants; five small CRSP plants differ in 2018-2020 (WAPA hourly data agrees with RISE) |

# 6. Corrected recording errors

All hourly generation values were screened for recording errors against independently measured flows (turbine flow for BPA, power release in the WAPA PDFs for CRSP). A value is corrected only when it is at least 5 times off the flow-implied output and a single recording error - one extra digit, or a decimal point misplaced by 10 or 100 - reproduces the flow-implied output within 5 %. Seven BPA values meet this rule. The published value stays in power_MW and the corrected value is in the added column power_MW_adjust (identical to power_MW in every other hour); BPA_dams/power_MW_adjustments.csv lists the changes. No CRSP value meets the rule.

| Dam | Hour (UTC) | power_MW (original) | power_MW_adjust | MW implied by flow | Recording error |
| --- | --- | --- | --- | --- | --- |
| CHJ | 2018-01-30 07:00 | 12,131.0 | 1,213 | 1,202 | delete digit '1' at position 5 |
| CHJ | 2018-02-17 16:00 | 10,992.0 | 1,992 | 1,986 | delete digit '0' at position 2 |
| CHJ | 2018-03-18 15:00 | 10,936.0 | 1,093 | 1,075 | delete digit '6' at position 5 |
| GCL | 2018-06-25 12:00 | 27,620.0 | 2,762 | 2,748 | delete digit '0' at position 5 |
| GCL | 2018-09-25 01:00 | 22,284.0 | 2,284 | 2,273 | delete digit '2' at position 1 |
| GCL | 2018-10-28 20:00 | 18.1 | 1,810 | 1,889 | decimal point misplaced (x100) |
| GCL | 2022-11-28 11:00 | 16.9 | 1,690 | 1,663 | decimal point misplaced (x100) |

All WAPA values (hourly and monthly) are kept exactly as published.

# 7. Known issues and limitations

Grand Coulee: power_MW is 8-12 % above USBR/EIA generation every year, although turbine flow matches USBR's water for generation exactly. The hourly pattern is usable; for absolute energy, scaling to USBR's monthly totals is recommended.

McNary is about 7 % above EIA-923 in 2021-2022; Hungry Horse (2018, 2019, 2021) and Dworshak (2018) differ by 3-7 %.

Seven obvious recording errors in the published BPA hourly data are corrected in power_MW_adjust (section 6).

About 11-12 % of hours are blank in the WAPA CRSP SCADA archive; blank hours are left empty.

Hourly generation after April 2022 has not been published by WAPA for the CRSP plants.

No public hourly or daily plant generation was found for Hoover, Davis, Parker, the Pick-Sloan plants or the Central Valley Project plants (sources checked: USBR RISE, USBR regional hydrodata pages, WAPA web site, CDEC, USACE Missouri River Basin bulletins); monthly data is provided for these.

The Pick-Sloan USACE mainstem dams are mostly in the Eastern Interconnection, not WECC (Fort Peck's units are split between the two grids).

RISE stores some months twice; the most recently created record is kept. Pairs with different values are noted in the notes column of WAPA_dams/sources.csv.