Verification report

Prepared by Yi Lu  |  Checks run 2026-10-02

Every check downloads the data again from the publishing agency; no data file is compared with itself.

# BPA dams (folder BPA_dams/)

## B1. Manual download links: full re-download and value-by-value comparison

| Dam | Columns | Values compared | Mismatches |
| --- | --- | --- | --- |
| ALF | 8 | 349,901 | 0 |
| BON | 9 | 394,265 | 0 |
| CHJ | 9 | 394,273 | 0 |
| DWR | 9 | 392,349 | 0 |
| GCL | 10 | 438,095 | 0 |
| HGH | 9 | 392,855 | 0 |
| IHR | 8 | 350,157 | 0 |
| JDA | 8 | 306,694 | 0 |
| LIB | 8 | 348,457 | 0 |
| LGS | 8 | 350,119 | 0 |
| LWG | 9 | 349,835 | 0 |
| LMN | 8 | 349,931 | 0 |
| MCN | 7 | 306,405 | 0 |
| TDA | 8 | 306,525 | 0 |

Total: 5,029,861 values compared, 0 mismatches.

## B2. Time stamps: CWMS The Dalles outflow vs USGS 14105700 (Jan-Feb 2020)

Best lag = 0 h (r = 0.87). A 0 h lag confirms the files are in UTC and hour-ending.

## B3. Annual energy vs EIA-923 plant net generation

Ratio = sum of hourly power_MW over the calendar year (Pacific time) / EIA-923 annual net generation. CWMS reports gross output and EIA net output, so ratios slightly above 1.00 are expected. Bold = more than 3 % from EIA.

| Dam | EIA plant id | 2018 | 2019 | 2020 | 2021 | 2022 |
| --- | --- | --- | --- | --- | --- | --- |
| ALF | 851 | 1.011 | 1.012 | 1.012 | 1.005 | 1.018 |
| BON | 3075 | 1.007 | 1.008 | 1.014 | 1.007 | 1.011 |
| CHJ | 3921 | 1.003 | 1.001 | 1.006 | 1.001 | 1.004 |
| DWR | 840 | 0.948 | 1.004 | 1.006 | 0.999 | 1.006 |
| GCL | 6163 | 1.114 | 1.079 | 1.106 | 1.107 | 1.116 |
| HGH | 2203 | 1.066 | 1.031 | 1.028 | 0.946 | 1.015 |
| IHR | 3925 | 1.012 | 1.013 | 1.014 | 1.007 | 1.004 |
| JDA | 3082 | 1.002 | 1.002 | 1.014 | 1.024 | 1.009 |
| LIB | 6172 | 0.972 | 1.003 | 1.001 | 1.009 | 1.000 |
| LGS | 3926 | 1.003 | 1.004 | 1.006 | 1.001 | 1.004 |
| LWG | 6175 | 1.005 | 1.006 | 1.006 | 1.000 | 1.006 |
| LMN | 3927 | 1.003 | 1.005 | 1.010 | 1.004 | 1.002 |
| MCN | 3084 | 0.995 | 1.008 | 1.004 | 1.071 | 1.068 |
| TDA | 3895 | 1.002 | 1.003 | 1.004 | 1.009 | 1.006 |

# WAPA dams (folder WAPA_dams/)

## W1. USBR RISE monthly series: full re-download through the download_url in sources.csv

| Project | Series | Values compared | Mismatches |
| --- | --- | --- | --- |
| 1_Boulder_Canyon | 4 | 240 | 0 |
| 2_Parker_Davis | 8 | 480 | 0 |
| 3_CRSP | 24 | 1,440 | 0 |
| 4_Pick_Sloan | 81 | 4,860 | 0 |
| 5_Central_Valley | 40 | 2,400 | 0 |

Units in the RISE CSV match the column units (kWh / kW) for every series: yes.

## W2. CRSP hourly SCADA generation vs USBR monthly gross generation

Only months in which every hour has a SCADA value are compared.

| Plant | Complete months | Median ratio | Min | Max |
| --- | --- | --- | --- | --- |
| BlueMesa | 25 | 1.000 | 1.000 | 1.000 |
| Crystal | 25 | 1.000 | 0.989 | 1.001 |
| FlamingGorge | 25 | 1.000 | 0.998 | 1.000 |
| Fontenelle | 25 | 1.000 | 0.998 | 1.003 |
| GlenCanyon | 25 | 1.000 | 1.000 | 1.000 |
| MorrowPoint | 25 | 1.000 | 0.987 | 1.030 |

## W3. CRSP SCADA clock: Flaming Gorge generation vs USGS 09234500 (just below the dam)

Winter (Jan-Feb 2019): best lag = 0 h (r = 0.98)

Summer (Jul-Aug 2019): best lag = 0 h (r = 0.98)

Same 0 h lag in winter and summer: the SCADA clock is Mountain Standard Time all year and the +7 h conversion to UTC is correct.

## W4. USBR RISE annual net generation vs EIA-923 (independent reporting systems)

EIA plant ids are matched by plant name and operator 'U S Bureau of Reclamation' in EIA-923. Bold = more than 3 % apart.

| Project | Plant | EIA plant id(s) | 2018 | 2019 | 2020 | 2021 | 2022 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1_Boulder_Canyon | Hoover | 154,8902 | 1.000 | 1.000 | 1.000 | 1.000 | 0.991 |
| 2_Parker_Davis | Davis | 152 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 2_Parker_Davis | Parker | 447 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 3_CRSP | Blue Mesa | 512 | 0.798 | 1.320 | 0.823 | 1.000 | 1.000 |
| 3_CRSP | Crystal | 6159 | 0.805 | 1.272 | 0.800 | 1.000 | 1.000 |
| 3_CRSP | Flaming Gorge | 6405 | 0.930 | 0.986 | 0.910 | 1.000 | 1.000 |
| 3_CRSP | Fontenelle | 4185 | 0.900 | 1.041 | 0.840 | 1.000 | 1.000 |
| 3_CRSP | Glen Canyon | 153 | 1.000 | 1.000 | 1.000 | 1.000 | 1.001 |
| 3_CRSP | Morrow Point | 514 | 0.834 | 1.179 | 0.856 | 1.000 | 1.000 |
| 4_Pick_Sloan | Alcova | 6409 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Big Thompson | 515 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Boysen | 505 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Buffalo Bill | 7317 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Canyon Ferry | 6400 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Estes | 513 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Flatiron Unit 3 (pump-generating) | 518 | 0.857 | 1.000 | 0.972 | 0.918 | 1.000 |
| 4_Pick_Sloan | Flatiron Units 1-2 | 518 | 1.012 | 1.114 | 1.000 | 1.025 | 1.000 |
| 4_Pick_Sloan | Fremont Canyon | 4176 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Glendo | 4177 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Green Mountain | 516 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Guernsey | 4178 | 1.000 | 0.998 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Heart Mountain | 6408 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Kortes | 4180 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Marys Lake | 517 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Pole Hill | 519 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Seminoe | 4182 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Shoshone | 4183 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Spirit Mountain | 7541 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 4_Pick_Sloan | Yellowtail | 2204 | 1.000 | 1.001 | 1.000 | 1.000 | 1.000 |
| 5_Central_Valley | Folsom | 441 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 5_Central_Valley | Judge Francis Carr | 442 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 5_Central_Valley | Keswick | 443 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 5_Central_Valley | Lewiston | not in EIA-923 |  |  |  |  |  |
| 5_Central_Valley | New Melones | 6158 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 5_Central_Valley | Nimbus | 444 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 5_Central_Valley | Shasta | 445 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 5_Central_Valley | Spring Creek | 450 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 5_Central_Valley | Stampede | 7066 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 5_Central_Valley | Trinity | 451 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |

# Recording-error screening

## Q1. Rule

Every hour of BPA power_MW was compared with the independently measured turbine flow, and every CRSP hour with the power release read again from the 48 WAPA PDFs.

A value is corrected only if it is an isolated value (neighbouring hours agree, flow / release steady), it is at least 5 times larger or smaller than the flow-implied output, and a single recording error - one extra digit, or a decimal point misplaced by 10 or 100 - reproduces the flow-implied output within 5 %. Where two reconstructions fit about equally well (within 1 % of each other), one is chosen and the other is listed.

## Q2. Corrected values (BPA, column power_MW_adjust)

| Dam | Hour (UTC) | power_MW (original) | power_MW_adjust | MW implied by flow | Recording error |
| --- | --- | --- | --- | --- | --- |
| CHJ | 2018-01-30 07:00 | 12,131 | 1,213 | 1,202 | extra final digit '1' (1,211 equally close) |
| CHJ | 2018-02-17 16:00 | 10,992 | 1,992 | 1,986 | extra digit '0' |
| CHJ | 2018-03-18 15:00 | 10,936 | 1,093 | 1,075 | extra digit '6' (1,096 equally close) |
| GCL | 2018-06-25 12:00 | 27,620 | 2,762 | 2,748 | extra final digit '0' (2,760 equally close) |
| GCL | 2018-09-25 01:00 | 22,284 | 2,284 | 2,273 | extra digit '2' |
| GCL | 2018-10-28 20:00 | 18.1 | 1,810 | 1,889 | decimal point (x100) |
| GCL | 2022-11-28 11:00 | 16.9 | 1,690 | 1,663 | decimal point (x100) |

No CRSP hourly value meets the rule; all CRSP values are kept as published.

power_MW keeps the published values; power_MW_adjust equals power_MW except these 7 hours (change log: BPA_dams/power_MW_adjustments.csv).

## Q3. Monthly records

RISE: all values kept as published. Internal inconsistencies noted only (no flag, no change): Lewiston (0.35 MW) reports net = 0 with gross > 0 in 19 months and gross above its reported capacity in some months; Nimbus reports negative station service in November 2021.

EIA-923 mainstem: no negative, missing or impossible values (maximum monthly capacity factor 1.15).

## Q4. Changes to the data files

Every original column of all 26 data files was compared with the files as downloaded: all identical, including power_MW; all WAPA files are byte-identical to the download. The added column power_MW_adjust differs from power_MW in exactly the 7 hours above.